5.h9LARGEST NUMBER IN AN ARRAY

EXP NO: 10

AIM: To find the largest number from an array using 8085 processor.

ALGORITHM:

- Load the address of the first element of the array in HL pair.

- Move the count to B register.

- Increment the pointer.

- Get the first data in A register.

- Decrement the count.

- Increment the pointer.

- Compare the content of memory addressed by HL pair with that of A register.

- If carry=0, go to step 10 or if carry=1 go to step 9

- Move the content of memory addressed by HL to A register.

- Decrement the count.

PROGRAM:

LXI H,2050

MOV C,M

DCR C

INX H

MOV A,M

LOOP1: INX H

CMP M

JNC LOOP

MOV A,M

LOOP: DCR C

JNZ LOOP1

STA 2058

HLT

OUTPUT:

RESULT: Thus the program was executed successfully using 8086 processor simulator.